FIGURE 11. 1

Bussing arrangement and system components.

FIGURE 11. 2

CPU interface.

FIGURE 11.3a

CPU read and write requests.

FIGURE 11. 3b

CPU read and write requests.

FIGURE 11. 4

Memory interface.

FIGURE 11. 5

Memory read operation.

FIGURE 11. 6

Controlling bus access.

FIGURE 11. 7

Bus grant for read operation.

USE basic_utilities.ALL;

--

ENTITY arbitrator IS

GENERIC (wait_states : natural_vector (3 DOWNTO 0) := (OTHERS => 1);

clock_period : TIME := 1 US);

PORT (read_request, write_request : IN nibble; grant : BUFFER nibble;

clock, skip_wait : IN qit; memsel, rwbar, ready : OUT qit);

END arbitrator;

--

ARCHITECTURE behavioral OF arbitrator IS

BEGIN

-- Works with consecuitive requests

wait_cycle: PROCESS

BEGIN

IF clock = '0' THEN

WAIT FOR 20 NS;

FOR i IN read_request'RANGE LOOP

IF read_request(i) = '1' OR write_request(i) = '1' THEN

grant <= "0000"; grant (i) <= '1';

memsel <= '1';

rwbar <= read_request (i);

ready <= '0';

IF wait_states (i) /= 0 THEN

wait: FOR j IN 1 TO wait_states (i) LOOP

EXIT WHEN skip_wait = '1';

WAIT FOR clock_period;

END LOOP wait;

END IF;

ready <= '1';

EXIT;

ELSE

grant (i) <= '0';

memsel <= '0';

END IF;

END LOOP;

END IF;

WAIT ON clock;

END PROCESS wait_cycle;

END behavioral;

--

FIGURE 11. 8

Arbiter VHDL description.

FIGURE 11. 9

Interface of serial-to-parallel converter.

FIGURE 11. 10

Interface of the DMA controller.

FIGURE 11. 11

DMA Registers.

FIGURE 11. 12

Decoding for selecting DMA registers.

USE basic_utilities.ALL;

--

ENTITY quad_adrdcd IS

GENERIC (addresses : twelve := "111111111100");

PORT (adbus : IN twelve; active : OUT qit; selects : OUT nibble);

END quad_adrdcd;

--

ARCHITECTURE behavioral OF quad_adrdcd IS

BEGIN

PROCESS (adbus)

BEGIN

IF to_bitvector (addresses AND adbus) = to_bitvector (addresses) THEN

active <= '1';

CASE adbus (1 DOWNTO 0) IS

WHEN "00" => selects <= "0001";

WHEN "01" => selects <= "0010";

WHEN "10" => selects <= "0100";

WHEN "11" => selects <= "1000";

WHEN OTHERS => selects <= "0000";

END CASE;

ELSE

active <= '0';

selects <= "0000";

END IF;

END PROCESS;

END behavioral;

FIGURE 11. 13

VHDL description of DMA register address decoder.

FIGURE 11. 14

DMA device.

ENTITY dma_controller IS

PORT (clk : IN qit;

-- memory signals

read_mem, write_mem : OUT qit := '0';

databus : INOUT byte := "ZZZZZZZZ"; adbus : INOUT twelve := "ZZZZZZZZZZZZ";

ready, grant : IN qit;

-- cpu signals

select_reg : IN nibble; status_rd, status_wr : IN qit;

--device signals

error1, error2, dev_rdy : IN qit;  dev_rcv : OUT qit;

dev_data : IN byte

);

END dma_controller;

FIGURE 11. 15

DMA controller entity declaration.

ARCHITECTURE behavioral of dma_controller IS

Declarations (Figure 11.17)

BEGIN

“get serial, put parallel” process statement (Figure 11.18)

--

“direct CPU communications” blocks (Figure 11.19)

END behavioral;

FIGURE 11. 16

Outline of DMA controller architecture.

SIGNAL done : qit := '0';

TYPE r4 IS ARRAY (0 TO 3) OF byte;

SIGNAL rfile : r4 REGISTER := (OTHERS => zero_8);

ALIAS go : qit IS rfile(3)(0);

ALIAS rd : qit IS rfile(3)(1);

ALIAS wr : qit IS rfile(3)(2);

ALIAS ie : qit IS rfile(3)(3);

FIGURE 11. 17

DMA controller declarations.

get_put : PROCESS

VARIABLE buff : byte := zero_8;

VARIABLE pntr : twelve;

VARIABLE numb : byte;

BEGIN

WAIT UNTIL go = '1';

done <= '0';

numb := rfile(2); pntr := rfile(1)(3 DOWNTO 0) & rfile(0);

IF wr = '1' THEN

writing : WHILE TO_INTEGER(numb) > 0 LOOP

numb    := numb - 1;

-- get data

IF dev_rdy /= '1' THEN WAIT UNTIL dev_rdy = '1'; END IF;

buff := dev_data;

WAIT UNTIL clk = '1';  dev_rcv <= '1';

WAIT UNTIL clk = '0';  dev_rcv <= '0';

-- put to mem

write_mem <= '1';

WAIT UNTIL grant = '1';

databus <= buff;

adbus <= pntr;

pntr := pntr + 1;

WAIT UNTIL ready = '1';

databus <= "ZZZZZZZZ";

adbus <= "ZZZZZZZZZZZZ";

write_mem <= '0';

END LOOP writing;

done <= '1';

END IF;

END PROCESS get_put;

FIGURE 11. 18

DMA controller “get serial and put parallel” process.

cpu_direct : FOR i IN 0 TO 3 GENERATE

databus <= rfile(i) WHEN select_reg(i) = '1' AND status_rd = '1' ELSE "ZZZZZZZZ";

r0to3 : BLOCK ((clk'EVENT AND clk = '0') AND select_reg(i) = '1' AND status_wr = '1')

BEGIN

rfile (i) <= GUARDED databus;

END BLOCK;

END GENERATE cpu_direct;

r3 : BLOCK ((clk'EVENT AND clk = '0') AND done = '1')

BEGIN

rfile (3)(7 DOWNTO 4) <= GUARDED ('1', ie, error2, error1);

END BLOCK;

FIGURE 11. 19

DMA controller “direct CPU communications” blocks.

--

ENTITY dma_serial_device IS

PORT (clk : IN qit;

-- memory signals

read_mem, write_mem : OUT qit := '0';

databus : INOUT byte := "ZZZZZZZZ"; adbus : INOUT twelve; ready, grant : IN qit;

status_rd, status_wr : IN qit; status_sel : OUT qit; serial_in : IN qit);

END dma_serial_device;

--

ARCHITECTURE structural OF dma_serial_device IS

COMPONENT dma IS

PORT (clk : IN qit; read_mem, write_mem : OUT qit;

databus : INOUT byte := "ZZZZZZZZ"; adbus : INOUT twelve; ready, grant : IN qit;

select_reg : IN nibble; status_rd, status_wr : IN qit;

error1, error2, dev_rdy : IN qit;  dev_rcv : OUT qit; dev_data : IN byte

);

END COMPONENT dma;

COMPONENT dcd IS

GENERIC (addresses : twelve := "1111111111XX");

PORT (adbus : IN twelve; active : OUT qit; selects : OUT nibble);

END COMPONENT dcd;

COMPONENT s2p IS

GENERIC (bps : INTEGER := 9600);

PORT (serial, received : IN qit; dataready : BUFFER qit;

overrun, frame_error : OUT qit;

parallel_out : BUFFER qit_vector (7 DOWNTO 0));

END COMPONENT s2p;

SIGNAL s2p_rdy, s2p_rcv, s2p_er1, s2p_er2 : qit;

SIGNAL s2p_par : byte;

SIGNAL cpu_mem_data : byte; SIGNAL cpu_mem_addr : twelve;

SIGNAL select_reg : nibble;

BEGIN

c1 : dma PORT MAP (clk, read_mem, write_mem, databus, adbus, ready, grant, select_reg,

status_rd, status_wr, s2p_er1, s2p_er2, s2p_rdy, s2p_rcv, s2p_par);

c2 : dcd PORT MAP (adbus, status_sel, select_reg);

c3 : s2p PORT MAP (serial_in, s2p_rcv, s2p_rdy, s2p_er1, s2p_er2, s2p_par);

END structural;

FIGURE 11. 20

DMA serial device, description for diagram of Figure 11.14.

FIGURE 11. 21

Cache Block Diagram.

FIGURE 11. 22

The lru table.

FIGURE 11. 23

Cache Interface.

ENTITY cache_system IS

PORT (clk : IN qit;

-- memory signals

read_mem, write_mem : OUT qit;

grant_mem, ready_mem : IN qit;

mem_databus : INOUT byte := "ZZZZZZZZ";

mem_adbus : INOUT twelve := "ZZZZZZZZZZZZ";

-- cpu signals

read, write : IN qit;

grant, ready : OUT qit;

databus : INOUT byte := "ZZZZZZZZ";

adbus : INOUT twelve := "ZZZZZZZZZZZZ"

);

END cache_system;

FIGURE 11. 24

Cache Entity Declaration.

ARCHITECTURE control_and_memory of cache_system IS

structure declarations

BEGIN

PROCESS

local declarations

BEGIN

wait for request

look for data in the cache

For read, write

If hit:

For read, pass data to CPU

For write, write data in cache and memory

If miss:

Find least recently used

For write, write data in cache and memory

For read, read from memory and pass on to CPU

Wait until (read OR write)=’1’;

END PROCESS;

END control_and_memory;

FIGURE 11. 25

Outline of cache VHDL description.

SUBTYPE ways IS INTEGER RANGE 0 TO 1;

SUBTYPE sets IS INTEGER RANGE 0 TO 31;

TYPE line IS ARRAY (0 TO 0) OF byte;

SUBTYPE tags IS qit_vector (6 DOWNTO 0);

TYPE lru_type IS ARRAY (sets) OF ways;

TYPE entry IS RECORD

valid : BOOLEAN;

tag : tags;

data : line;

END RECORD;

TYPE each_cache IS ARRAY (sets) OF entry;

TYPE cache_type IS ARRAY (ways) OF each_cache;

SIGNAL cache : cache_type;

SIGNAL lru : lru_type;

FIGURE 11. 26

Cache structure declarations.

VARIABLE s : sets;

VARIABLE hit : BOOLEAN;  VARIABLE w, free : ways;

TYPE ww IS ARRAY(ways) OF ways;

CONSTANT nw : ww := (1, 0);

ALIAS set_value : qit_vector (4 DOWNTO 0) IS adbus (4 DOWNTO 0);

ALIAS tag_value : tags IS adbus (11 DOWNTO 5);

FIGURE 11. 27

Controller local declarations.

grant <= '1';  ready <= '0';

WAIT UNTIL clk = '0';

s := TO_INTEGER (set_value);

hit := FALSE;

FOR i IN ways LOOP

IF cache(i)(s).tag = tag_value AND cache(i)(s).valid THEN

hit := TRUE;  w := i;

END IF;

END LOOP;

FIGURE 11. 28

Controller search in cache.

IF hit THEN

lru (s) <= nw (w);

IF read = '1' THEN

ready <= '1';

databus <= cache(w)(s).data(0);

WAIT UNTIL read = '0';

databus <= "ZZZZZZZZ";

ELSIF write = '1' THEN

cache(w)(s).data(0) <= databus;

cache(w)(s).valid <= TRUE;

write_mem <= '1';

WAIT UNTIL grant_mem = '1';

mem_databus <= databus;  mem_adbus <= adbus;

WAIT UNTIL ready_mem = '1';

mem_databus <= "ZZZZZZZZ";  mem_adbus <= "ZZZZZZZZZZZZ";  write_mem <= '0';

ready <= '1';

WAIT UNTIL write = '0';

END IF;

ready <= '0';

FIGURE 11. 29

Controller code for cache hit.

ELSE -- miss

free := lru (s);

lru (s) <= nw (lru (s));

IF write = '1' THEN

cache(free)(s).tag <= tag_value;

cache(free)(s).data(0) <= databus;

cache(free)(s).valid <= TRUE;

write_mem <= '1';

WAIT UNTIL grant_mem = '1';

mem_databus <= databus;  mem_adbus <= adbus;

WAIT UNTIL ready_mem = '1';

mem_databus <= "ZZZZZZZZ";  mem_adbus <= "ZZZZZZZZZZZZ";  write_mem <= '0';

ready <= '1';

WAIT UNTIL write = '0';

ready <= '0';

ELSIF read = '1' THEN

read_mem <= '1';

WAIT UNTIL grant_mem = '1';

mem_adbus <= adbus;

WAIT UNTIL ready_mem = '1';

cache(free)(s).tag <= tag_value;

cache(free)(s).data(0) <= mem_databus;

cache(free)(s).valid <= TRUE;

databus <= mem_databus;

mem_adbus <= "ZZZZZZZZZZZZ";  read_mem <= '0';

ready <= '1';

WAIT UNTIL read = '0';

ready <= '0';

END IF;

END IF;

FIGURE 11. 30

Controller code for cachemiss.

FIGURE 11. 31

Board level interface.

ENTITY parwan_tester IS END parwan_tester;

--

ARCHITECTURE system OF parwan_tester IS

BEGIN

int : interrupt <= '1', '0' AFTER 4500 NS;

clk : clock <= NOT clock AFTER duty WHEN halted = '0' ELSE clock;

arb : arbitr GENERIC MAP ((OTHERS => 2), period)

PORT MAP (rd_req, wr_req, grant_mem, clock, skip_wait, cs, rwbar, ready);

dev : serial PORT MAP (clock, rd_req(0), wr_req(0), data, address, ready, grant_mem(0),

cpu_read, cpu_write, skip_wait, serial_in);

csh : cache  PORT MAP (clock, rd_req(1), wr_req(1), grant_mem(1), ready, data, address,

cpu_read, cpu_write, csh_grant, csh_ready, cpu_data, cpu_address);

cpu : parwan PORT MAP (clock, interrupt, cpu_read, cpu_write, cpu_data, cpu_address,

halted, csh_ready, csh_grant);

mem : memory PORT MAP (cs, rwbar, data, address);

srg : sergen PORT MAP (serial_in);

END system;

FIGURE 11. 32

Interface board VHDL description.

FIGURE 11. 33

Interface board simulation run.