-------------------------------------------------------------------------------

--

-- Floating Point Multiplier Benchmark: Main File Documentation

--

-- Source:  Patterson, David A., and Hennessy, John L.,  "Computer

--	         architecture: a quantitative approach".  San Mateo, CA: Morgank

--		 Kaufman Publishers, 1990,  Appendix A, p. 3-22

--

-- Author: Jesse Pan

--         Department of Electrical and Computer Engineering

--         University of California, Irvine, CA 92717

--

-- Acknowledgement: Special thanks to Dr. Tomas Lang's advice on this benchmark

--

-- Written on Mar 01, 1994

--

-------------------------------------------------------------------------------

1) INTRODUCTION:

The Floating Point Multiplier is a design algorithm that performs an

operation, given the operator and two IEEE standard source operands, and

produces an IEEE standard result.

Both the two inputs and the output are represented by a sign bit, a 127-biased

integer exponent in the range 0..255, and a 23-bit vector mantissa with a

"hidden 1"(e.g. implicit representation in Patterson and Hennessy, "Computer

Architecture: a Quantitative Approach").  This is the IEEE 754 standard, and

its representation is shown below:

______________________________

| sign | exponent | mantissa |

|______|__________|__________|

1 bit   8 bits     23 bits

------------------------------------------------------------------------------

(1.1)  FUNCTIONAL BLOCK(S)

The Multiplier has one main functional block, shown below:

** The Main functional block consists of one process: the Main process.

## Main process describes the input and output fields, including the

respective signs, exponents, and mantissas.

OPERAND 1   OPERAND 2

|           |

______v___________v______

|                       |

OPERATION --->|     FLOATING POINT    |

|    ADDER/SUBTRACTOR   |

CLOCK --->|_______________________|

|         |

v         v

RESULT    FLAGS

------------------------------------------------------------------------------

(1.2)  PORTS

INPUT AND OUTPUT PORTS:

______________________________________________________________________________

|   PORTS   | TYPE | BIT   | DESCRIPTION				      |

|           |      | WIDTH |						      |

|-----------|------|-------|--------------------------------------------------|

|    clk    |  in  |   1   | internal timing				      |

|-----------|------|-------|--------------------------------------------------|

| op1_sign  |  in  |   1   | bit sign of 1st operand			      |

|-----------|------|-------|--------------------------------------------------|

|  op1_exp  |  in  |   8   | integer exponent of 1st operand, in range 0..255 |

|-----------|------|-------|--------------------------------------------------|

| op1_mant  |  in  |  23   | 23-bit vector mantissa of 1st operand	      |

|-----------|------|-------|--------------------------------------------------|

| op2_sign  |  in  |   1   | bit sign of 2nd operand			      |

|-----------|------|-------|--------------------------------------------------|

|  op2_exp  |  in  |   8   | integer exponent of 2nd operand, in range 0..255 |

|-----------|------|-------|--------------------------------------------------|

| op2_mant  |  in  |  23   | 23-bit vector mantissa of 2nd operand	      |

|-----------|------|-------|--------------------------------------------------|

| res_sign  |  out |   1   | bit sign of result				      |

|-----------|------|-------|--------------------------------------------------|

|  res_exp  |  out |   8   | integer exponent of result, in range 0.255       |

|-----------|------|-------|--------------------------------------------------|

| res_mant  |  out |  23   | 23-bit vector mantissa of result		      |

|-----------|------|-------|--------------------------------------------------|

| operation |  in  |  ---  | signal that indicates operation, with possible   |

|           |      |       | values: multiply, idle                           |

|-----------|------|-------|--------------------------------------------------|

|   flags   |  out |   4   | 4-bit vector that sets status bit flags          |

|	    |      |       | flag(3)=zero 	flag(2)=pos_inf	              |

|	    |      |       | flag(1)=neg_inf    flag(0)=Nan

|___________|______|_______|__________________________________________________|

-------------------------------------------------------------------------------

(1.3)  THE INSTRUCTION SET:

The Floating Point Adder has three instructions as follows:

** multiply, idle:  control performing operation and producing result

values based on input operands

(1.3.0) The multiplier does not handel denormal floating point number.

If result of the multiplication are

(1.3.1) Overflow/Underflow result exponent

If the result exponent of the multiplication are overflow(exp>255) or

underflow(exp<0) then the floating point multiplier will give warning

and the output of the multiplier will have no effect from the

operation.

-------------------------------------------------------------------------------

(1.3.2)  FUNCTIONS:

_____________________________________________________________________________

|  OPERATION  |  EFFECT UPON RESULT 				              |

|             |								      |

|=============================================================================|

|  multiply   | sets result's sign, exponent, and mantissa after "multiply"   |

|	      | operands                                	              |

|-------------|---------------------------------------------------------------|

|    idle     | maintains previous result's sign, exponent, and mantissa      |

|_____________|_______________________________________________________________|

(1.3.3) Special input operation table

_________________________________________

|     special operands operation table    |

|-----------------------------------------|

|  op1\op2|| Zero  | Nan |  +Inf  | -Inf  |

|=========================================|

|   zero  || Zero  | Nan |  Zero  | Zero  |

|-----------------------------------------|

|   Nan   || Nan   | Nan |  Nan   | Nan   |

|-----------------------------------------|

|   +Inf  || Zero  | Nan |  +Inf  | -Inf  |

|-----------------------------------------|

|   -Inf  || Zero  | Nan |  -Inf  | +Inf  |

|_________||_______|_____|________|_______|

===============================================================================

2)  MODEL DEVELOPED FOR THE Floating Point Multiplier

main process -> The Multiplier is modeled as a single VHDL process

===============================================================================

3)  TESTING STRATEGY

The functions above were tested with the set of test vectors in the file:

"test.vhd"

The test vectors were chosen to exhaustively test both nominal and boundary

values of the sign, exponent, and mantissa of both source operands

Note:  The exponents used are biased-127, in other words, the actual exponent

is 127 less than its representation

Also, the mantissa is represented in "hidden 1" representation

Nominal Operand:  Sign = 0 or 1,  Exponent = 1 to 254

Mantissa = any 23-bit combination of 0's and 1's, with the

exception of all 0's

The rounding conditions are tested, the rounding mode is based

on IEEE 754 floating-point standard default mode(round/even).

Boundary Operands:

Zero:  Sign = 0 or 1,  Exponent = 0,

Mantissa = 23-bit vector consisting of all 0's

Minimum Positive:  Sign = 0,  Exponent = 0

Mantissa = 23 bit-vector: 00000..01

Minimum Negative:  Sign = 1,  Exponent = 0

Mantissa = 23 bit-vector: 00000..01

Maximum Positive:  Sign = 0,  Exponent = 254

Mantissa = 23-bit vector consisting of all 1's

Maximum Negative:  Sign = 1,  Exponent = 254

Mantissa = 23-bit vector consisting of all 1's

Positive Infinity: Sign = 0,  Exponent = 255

Mantissa = 23-bit vector consisting of all 0's

Negative Infinity: Sign = 1,  Exponent = 255

Mantissa = 23-bit vector consisting of all 0's

Not a Number(NaN): Sign = 0 or 1,  Exponent = 255

Mantissa = 23-bit vector consisting of any

23-bit combination of 0's and 1's, with the

exception of all 0's

Each of the three functions was thoroughly tested for correctness and

precision.

===============================================================================

4)  STATUS OF MODELS

____________________________________________________

|         |                  |              |        |

| MODEL   | TEST VECTOR USED |   SIMULATOR  | ERRORS |

|_________|__________________|______________|________|

|  main   |    		     |	   	    |  	     |

| process |    test.vhd      | Synopsys 3.0a|  None  |

|_________|__________________|______________|________|

<div align="center"><br /><script type="text/javascript"><!--

google_ad_client = "pub-7293844627074885";

//468x60, Created at 07. 11. 25

google_ad_slot = "8619794253";

google_ad_width = 468;

google_ad_height = 60;

//--></script>

<script type="text/javascript" src="http://pagead2.googlesyndication.com/pagead/show_ads.js">

</script><br />&nbsp;</div>